## Requirements:

- Arduino IDE (download from http://www.arduino.cc)
- Arduino Mega 2560
- PuTTY Terminal (download from http://putty.org/)
- [Optional] LCD/Keyboard Shield for Arduino

## Steps:

0. Unzip the Arduino code to your preferred location.

Upload Arduino Code:

1. Connect Arduino Mega to computer.
2. Start Arduino IDE and select correct Arduino COM PORT under menus: (Tools / Port). 
3. Open "kz80_efex.ino" and compile it (From menus: Sketch / Verify-Compile).
4. Upload sketch next (From menus: Sketch / Upload).

Plug RetroShield:

5. Disconnect Arduino Mega from computer.
6. Plug RetroShield to Arduino Mega.
7. Ensure header pins are aligned correctly. 
8. Connect Arduino Mega back to computer.

Start Terminal Program:

9.  Start Putty Terminal Program.
10. Find the Arduino COM port from Arduino IDE, menu Tools/Ports.  (You will see something like COM3, etc.). Enter this number in the Serial Line entry:

Serial Line = COM3
Speed = 115200
Connection Type = Serial

11. Click Open.
12. You should see the initial message from monitor program:

EfexMon By M.PEKER v1.0b
Press H for help

Ready.

C:>

13.  type H to print a list of available commands (need to type uppercase for now):

A-Assembler
D-Disassembler
L-Load
S-Save
E-Edit
W-Warmst
M-Move
X-Hexdump
G-Go
F-Fill
U-Routines

14. type A then F000
15. type NOP [ENTER] NOP [ENTER] RET [ENTER].  Be patient after pressing ENTER.
16. hit ESCAPE
17. type G then F000
18. you just executed your first program :)